//=========================================

//Created by    :lidong

//Filename      :afifo_addr_gen.v

//Author        :lidong(RDC)

//Created On    :2024-07-01 21:14

//Last Modified :

//Update Count  :

//Description   :

//

//

//=========================================

//

module afifo_addr_gen #(

parameter  integer DW_IN  = 10,

parameter  integer DW_OUT = 12

)

(

//=========================================

//ports declared by Verilog-mode tool

//=========================================

/*autoinput("^clk\|^rst")*/

/*autoinput("^din\([0-1]\)_\(a\|b\)")*/

/*autooutput("^dout_*")*/

//=========================================

// ports declared by user

//=========================================

// input/output to/from internal logic block

input                 din_vld,

output                dout_vld,

// output from inter-connection of internal instance

output  [11:0]        inter_out0_a,

output  [11:0]        inter_out0_b,

);

//===========================

//Local parameters

//===========================

//

//===========================

//Local wires and registers

//===========================

/*autologic*/

/*autodefine*/

//  User define

wire    userdef0;

wire    userdef1;

//  End of user define

//

//===========================

//Main code begins here

//===========================

//

/*

module0 auto_template (

.clk           (clk             ),

.rst_n         (rst_n           ),

.din_vld       (inter_vld@      ),

.din_a         (din@_a[]        ),

.din_b         (din@_b[]        ),

.dout\(.*\)  (inter_out@\1[] ),

.debug         (inter_con       ),

);

*/

module0 # (

.DW_IN    (DW_IN ),

.DW_OUT   (DW_OUT),

)  inst0 (/*autoinst*/);

//=========================================

//Verilog-mode Setting:

//Local Variables:

//verilog-library-files: (

//"./test.v"

//)

//verilog-library-directories: (

//"."

//".."

//)

//verilog-auto-inst-param-value: t

//End:

//=========================================

endmodule

选中空白行，再执行

自动添加块语句

手动编辑后的代码：

//=========================================

//Created by    :xxx

//Filename      :afifo_addr_gen.v

//Author        :xxx(RDC)

//Created On    :2024-07-01 21:14

//Last Modified :

//Update Count  :

//Description   :

//

//

//=========================================

//

module afifo_addr_gen #(

parameter  integer AW  = 4  //The width of address

)

(

//=========================================

//ports declared by Verilog-mode tool

//=========================================

/*autoinput("^clk\|^rst")*/

/*autoinput("^din\([0-1]\)_\(a\|b\)")*/

/*autooutput("^dout_*")*/

//=========================================

// ports declared by user

//=========================================

// input/output to/from internal logic block

input                 srst,

input                 en,           //write or read enable signal

output                flag,         //full or empty flag

// output from inter-connection of internal instance

output  [AW:0]      bptr,

output  [AW:0]      gptr,

output  [AW:0]      bptr_next,

output  [AW:0]      gptr_next

);

//===========================

//Local parameters

//===========================

//

//===========================

//Local wires and registers

//===========================

/*autologic*/

/*autodefine*/

//  User define

wire    userdef0;

wire    userdef1;

//  End of user define

//

//===========================

//Main code begins here

//===========================

//

assign   ram_en        = (en & (~flag));

assign   bptr_next[AW:0] = bptr[AW:0] + ram_ren;

assign   gptr_next[AW:0] = {1'd0,bptr_next[AW:1]}^bptr_next[AW:0];

always@( posedge clk or negedge rst_n ) begin

if( !rst_n) begin

bptr[AW:0] <= {(AW+1){1'b0}};

gptr[AW:0] <= {(AW+1){1'b0}};

end else if( srst ) begin

bptr[AW:0] <= {(AW+1){1'b0}};

gptr[AW:0] <= {(AW+1){1'b0}};

end else begin

bptr[AW:0] <= bptr_next[AW:0];

gptr[AW:0] <= gptr_next[AW:0];

end

end

//=========================================

//Verilog-mode Setting:

//Local Variables:

//verilog-library-files: (

//"./test.v"

//)

//verilog-library-directories: (

//"."

//".."

//)

//verilog-auto-inst-param-value: t

//End:

//=========================================

endmodule

进行V2AA后

端口输出是寄存器类型的，自动补充logic

自动定义中间信号

最终手动补齐其它代码：

//=========================================

//Created by    :xxx

//Filename      :afifo_addr_gen.v

//Author        :xxx(RDC)

//Created On    :2024-07-01 21:14

//Last Modified :

//Update Count  :

//Description   :

//

//

//=========================================

//

module afifo_addr_gen #(

parameter  integer AW  = 4  //The width of address

)

(

//=========================================

//ports declared by Verilog-mode tool

//=========================================

/*autoinput("^clk\|^rst")*/

input                       clk,

input                       rst_n,

/*autoinput("^din\([0-1]\)_\(a\|b\)")*/

/*autooutput("^dout_*")*/

//=========================================

// ports declared by user

//=========================================

// input/output to/from internal logic block

input                       srst,

input                       en,           //write or read enable signal

output                      flag,         //full or empty flag

// output from inter-connection of internal instance

output logic  [AW:0]        bptr,

output logic  [AW:0]        gptr,

output        [AW:0]        bptr_next,

output        [AW:0]        gptr_next

);

//===========================

//Local parameters

//===========================

//

//===========================

//Local wires and registers

//===========================

/*autologic*/

/*autodefine*/

//  Define flip-flop registers here

//  Define combination registers here

//  Define wires here

wire             ram_en;    //

//  End of automatic define

//  User define

//  End of user define

//

//===========================

//Main code begins here

//===========================

assign   ram_en          = (en & (~flag));

assign   bptr_next[AW:0] = bptr[AW:0] + ram_ren;

assign   gptr_next[AW:0] = {1'd0,bptr_next[AW:1]}^bptr_next[AW:0];

always@( posedge clk or negedge rst_n ) begin

if( !rst_n) begin

bptr[AW:0] <= {(AW+1){1'b0}};

gptr[AW:0] <= {(AW+1){1'b0}};

end else if( srst ) begin

bptr[AW:0] <= {(AW+1){1'b0}};

gptr[AW:0] <= {(AW+1){1'b0}};

end else begin

bptr[AW:0] <= bptr_next[AW:0];

gptr[AW:0] <= gptr_next[AW:0];

end

end

//=========================================

//Verilog-mode Setting:

//Local Variables:

//verilog-library-files: (

//"./test.v"

//)

//verilog-library-directories: (

//"."

//".."

//)

//verilog-auto-inst-param-value: t

//End:

//=========================================

Endmodule

//=========================================

//Created by    :xxx

//Filename      :afifo_wctrl.v

//Author        :xxx(RDC)

//Created On    :2024-07-02 20:11

//Last Modified :

//Update Count  :

//Description   :

//

//

//=========================================

//

module afifo_wctrl #(

parameter  integer DW_IN  = 10,

parameter  integer DW_OUT = 12

)

(

//=========================================

//ports declared by Verilog-mode tool

//=========================================

/*autoinput("^clk\|^rst")*/

/*autoinput("^din\([0-1]\)_\(a\|b\)")*/

/*autooutput("^dout_*")*/

//=========================================

// ports declared by user

//=========================================

// input/output to/from internal logic block

input                 din_vld,

output                dout_vld,

// output from inter-connection of internal instance

output  [11:0]        inter_out0_a,

output  [11:0]        inter_out0_b,

);

//===========================

//Local parameters

//===========================

//

//===========================

//Local wires and registers

//===========================

/*autologic*/

/*autodefine*/

//  User define

wire    userdef0;

wire    userdef1;

//  End of user define

//

//===========================

//Main code begins here

//===========================

//

/*

module0 auto_template (

.clk           (clk             ),

.rst_n         (rst_n           ),

.din_vld       (inter_vld@      ),

.din_a         (din@_a[]        ),

.din_b         (din@_b[]        ),

.dout\(.*\)  (inter_out@\1[] ),

.debug         (inter_con       ),

);

*/

module0 # (

.DW_IN    (DW_IN ),

.DW_OUT   (DW_OUT),

)  inst0 (/*autoinst*/);

//=========================================

//Verilog-mode Setting:

//Local Variables:

//verilog-library-files: (

//"./test.v"

//)

//verilog-library-directories: (

//"."

//".."

//)

//verilog-auto-inst-param-value: t

//End:

//=========================================

endmodule

选择空白处，执行

手动编辑后的代码：

//=========================================

//Created by    :xxx

//Filename      :afifo_wctrl.v

//Author        :xxx(RDC)

//Created On    :2024-07-02 20:11

//Last Modified :

//Update Count  :

//Description   :

//

//

//=========================================

//

module afifo_wctrl #(

parameter  integer AW      = 4,

parameter  integer ALFULL  = 12,

)

(

//=========================================

//ports declared by Verilog-mode tool

//=========================================

/*autoinput("^clk\|^rst")*/

input                 wclk,

input                 wrst_n,

input                 wsrst,

/*autoinput("^din\([0-1]\)_\(a\|b\)")*/

/*autooutput("^dout_*")*/

//=========================================

// ports declared by user

//=========================================

// input/output to/from internal logic block

input                 winc,

output  [AW:0]        r2w_gptr,

// output from inter-connection of internal instance

output  [AW:0]        wgptr,  // Gray write address for sync to read clock domain

output  [AW-1:0]      waddr,  // Write address for memory

output                wfull,  // Write full flag

output                wafull  // Write almost full flag

);

//===========================

//Local parameters

//===========================

//

//===========================

//Local wires and registers

//===========================

/*autologic*/

/*autodefine*/

//  User define

//  End of user define

//

//===========================

//Main code begins here

//===========================

// 写指针产生

assign   ram_wen          = (winc & (~wfull));     //非满且写的时候，产生memory写使能

assign   wbptr_next[AW:0] = wbptr[AW:0] + ram_wen; //非满且写的时候，写指针加一

assign   wgptr_next[AW:0] = {1'd0,wbptr_next[AW:1]}^wbptr_next[AW:0]; //写指针二进制转格雷码

always@( posedge wclk or negedge wrst_n ) begin

if( !wrst_n) begin

wbptr[AW:0] <= {(AW+1){1'b0}};

wgptr[AW:0] <= {(AW+1){1'b0}};

end else if( wsrst) begin

wbptr[AW:0] <= {(AW+1){1'b0}};

wgptr[AW:0] <= {(AW+1){1'b0}};

end else begin

wbptr[AW:0] <= wbptr_next[AW:0];

wgptr[AW:0] <= wgptr_next[AW:0];

end

end

// 满/将满标志产生

assign  wfull_tmp  = (wgptr_next[AW:0] == {~r2w_gptr[AW:AW-1],r2w_gptr[AW-2:0]});

assign  awfull_tmp = ((wbptr_next[AW:0] - r2w_gptr[AW:0]) > ALFULL);

always@( posedge wclk or negedge wrst_n ) begin

if( !wrst_n) begin

wfull  <= 1'b0;

awfull <= 1'b0;

end else if( wsrst) begin

wfull  <= 1'b0;

awfull <= 1'b0;

end else begin

wfull  <= wfull_tmp;

awfull <= awfull_tmp;

end

end

//=========================================

//Verilog-mode Setting:

//Local Variables:

//verilog-library-files: (

//"./test.v"

//)

//verilog-library-directories: (

//"."

//".."

//)

//verilog-auto-inst-param-value: t

//End:

//=========================================

endmodule

执行完后：

//=========================================

//Created by    :xxx

//Filename      :afifo_wctrl.v

//Author        :xxx(RDC)

//Created On    :2024-07-02 20:11

//Last Modified :

//Update Count  :

//Description   :

//

//

//=========================================

//

module afifo_wctrl #(

parameter  integer AW      = 4,

parameter  integer ALFULL  = 12,

)

(

//=========================================

//ports declared by Verilog-mode tool

//=========================================

/*autoinput("^clk\|^rst")*/

input                 wclk,

input                 wrst_n,

input                 wsrst,

/*autoinput("^din\([0-1]\)_\(a\|b\)")*/

/*autooutput("^dout_*")*/

//=========================================

// ports declared by user

//=========================================

// input/output to/from internal logic block

input                 winc,

output  [AW:0]        r2w_gptr,

// output from inter-connection of internal instance

output logic  [AW:0]        wgptr,  // Gray write address for sync to read clock domain

output        [AW-1:0]      waddr,  // Write address for memory

output logic  [AW-1:0]      waddr,  // Write address for memory

output                      wafull  // Write almost full flag

);

//===========================

//Local parameters

//===========================

//

//===========================

//Local wires and registers

//===========================

/*autologic*/

/*autodefine*/

//  Define flip-flop registers here

reg              awfull;    //

reg   [AW:0]     wbptr;    //

reg              wfull;    //

//  Define combination registers here

//  Define wires here

wire             awfull_tmp;    //

wire             ram_wen;    //

wire  [AW:0]     wbptr_next;    //

wire             wfull_tmp;    //

wire  [AW:0]     wgptr_next;    //

//  End of automatic define

//  User define

//  End of user define

//

//===========================

//Main code begins here

//===========================

// 写指针产生

assign   ram_wen          = (winc & (~wfull));     //非满且写的时候，产生memory写使能

assign   wbptr_next[AW:0] = wbptr[AW:0] + ram_wen; //非满且写的时候，写指针加一

assign   wgptr_next[AW:0] = {1'd0,wbptr_next[AW:1]}^wbptr_next[AW:0]; //写指针二进制转格雷码

always@( posedge wclk or negedge wrst_n ) begin

if( !wrst_n) begin

wbptr[AW:0] <= {(AW+1){1'b0}};

wgptr[AW:0] <= {(AW+1){1'b0}};

end else if( wsrst) begin

wbptr[AW:0] <= {(AW+1){1'b0}};

wgptr[AW:0] <= {(AW+1){1'b0}};

end else begin

wbptr[AW:0] <= wbptr_next[AW:0];

wgptr[AW:0] <= wgptr_next[AW:0];

end

end

// 满/将满标志产生

assign  wfull_tmp  = (wgptr_next[AW:0] == {~r2w_gptr[AW:AW-1],r2w_gptr[AW-2:0]});

assign  awfull_tmp = ((wbptr_next[AW:0] - r2w_gptr[AW:0]) > ALFULL);

always@( posedge wclk or negedge wrst_n ) begin

if( !wrst_n) begin

wfull  <= 1'b0;

awfull <= 1'b0;

end else if( wsrst) begin

wfull  <= 1'b0;

awfull <= 1'b0;

end else begin

wfull  <= wfull_tmp;

awfull <= awfull_tmp;

end

end

//=========================================

//Verilog-mode Setting:

//Local Variables:

//verilog-library-files: (

//"./test.v"

//)

//verilog-library-directories: (

//"."

//".."

//)

//verilog-auto-inst-param-value: t

//End:

//=========================================

endmodule